Table 133.5.3.6.1 Continuous Operations Routes

Type 5 Winter Event:  Frost, Flurries, Freezing Fog, Blowing Snow & Refreeze

| PAVEMENT TEMPERATURE RANGE AND TREND | TRAFFIC CONDITION | INITIAL OPERATION |  |  | SUBSEQUENT OPERATIONS |  |  | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Maintenance action | Spread rates |  | Maintenance action | Spread rates |  |  |
|  |  |  | Pre-wetted solid salt (lb/ln-mi) | Brine (gal/ln-mi) |  | Pre-wetted solid salt (lb/ln-mi) | Brine (gal/ln-mi) |  |
| Above 32ºF steady or rising | Any level | None, see comments |  |  | None, see comments |  |  | 1) Monitor pavement temperature closely; begin treatment if starts to fall to 32ºF and below and is at or below dew point |
| 28 to 32ºF, remaining in range or falling to 32ºF or below, and equal to or below dew point | Traffic rate less than 100 vehicles per hr | Apply brine or pre-wetted solid salt | 25-65 | 11-28 | Reapply pre-wetted solid salt as needed | 25-65 |  | 1) Monitor pavement closely; if pavement becomes wet or if thin ice forms, reapply salt at higher indicated rate. 2) Do not apply brine on ice so thick that the pavement cannot be seen |
|  | Traffic rate greater than 100 vehicles per hr | Apply brine or pre-wetted solid salt | 25-65 | 11-28 | Reapply brine pre-wetted solid salt as needed | 25-65 | 11-28 |  |
| 20 to 28ºF, remaining in range and equal to or below dew point | Any level | Apply brine or pre-wetted solid salt | 65-130 | 28-57 | Reapply brine pre-wetted solid salt as needed | 65-130 | 28-57 | 1) Monitor pavement closely; if thin ice forms, reapply salt at higher indicated rate 2) Applications will need to be more frequent at higher levels of condensation; if traffic volumes are not enough to disperse condensation, it may be necessary to increase frequency 3) It is not advisable to apply a brine at the indicated spread rate when the pavement temperature drops below 20ºF |
| 10 to 20ºF, remaining in range and equal to or below dew point | Any level | Apply pre-wetted solid salt | 130-200 |  | Reapply pre-wetted solid salt as needed | 130-200 |  | 1) Monitor pavement closely; if thin ice forms, reapply salt at higher indicated rate 2) Applications will need to be more frequent at higher levels of condensation; if traffic volumes are not enough to disperse condensation, it may be necessary to increase frequency |
| Below 10ºF, steady or falling | Any level | Apply abrasives |  |  | Apply abrasives as needed |  |  | 1) Monitor pavement closely, salt will have limited melting power in this temperature range. 2) Liquid calcium chloride may be used for pre-wetting salt/abrasive mix at colder temperatures. |

Notes:  TIMING.  (1) Conduct initial operation in advance of freezing.  Apply brine up to 3 hr in advance.  Use longer advance times in this range to effect drying when traffic volume is low.  Apply pre-wetted solid salt 1 to 2 hr in advance.  (2) In the absence of precipitation, brine at 33 gal/lane-mi has been successful in preventing bridge deck icing when placed up to 4 days before freezing on higher volume roads and 7 days before on lower volume roads.