Calibration form

A.  Multiple nozzle spray bars  - Even distribution.

1. 	Time water is caught________seconds.

2.	Nozzle 1 ___________oz.	       Nozzle 7 __________oz.

Nozzle 2 ___________oz.             Nozzle 8 __________oz.

Nozzle 3 ___________oz.             Nozzle 9 __________oz.

Nozzle 4 ___________oz.             Nozzle 10 _________oz.

Nozzle 5 ___________oz.             Nozzle 11 _________oz.

Nozzle 6 ___________oz. 	       Nozzle 12 _________oz.

3.	Total       ___________oz.

4.        Divide total oz. by # of nozzles.

___________oz. / ________nozzles = ___________Avg. output

5.	Figure 5% error of operating range. Any nozzle not performing within this range (+ or -) should be checked and replaced if needed.

__________Avg. output x   .05   = __________ 5% error.

B.  Gathering Information

1.	Desired speed ____________mph

a.	Truck#_____________

b.	Gear ____________ High  /  Low (circle)

c.	RPM_____________

2.	Operating pressure___________psi

3.	Test distance____________ft.

4.	Time to travel test distance ___________seconds

- Catch water for same time took to travel test distance at desired pressure.

__________ounces used divided by 128 oz. = _________ gallons used.

# Calibration form

(cont’d)

C.  Gallons per acre (GPA)

1.	Test Area ________ =    Test Distance________ X Pattern Width_

Acre (43,560)

2.	Gallons Per Acre(GPA)  ____________= Gallons Used ______________

Test Area

D.  Convert miles to acres.  Multiply miles to spray by the # of acres in a mile at desired width.

1.	_________ miles  x  2 (both sides)  =  _____________ miles

2.	_________miles  x  _________ acres/mile  = ___________ acres to spray

Area covered to Acres per mile.

Width(ft)	                   Acres/mile

2			  .24

4			  .48

6			  .72

8			  .97

10			1.21

12			1.45

16			1.93

20			2.42

E.  Calculate gallons needed to complete spray operation.

_________GPA  x   __________acres  = ____________  gallons needed

F.  Calculate amount of herbicide to add to tank or needed for whole job.

________acres  x  ________rate per acre  = ___________product needed

G.  Calculate acres per tank.

_____(Tank capacity gallons)/ ____ (gallons per acre)   =  ____ acres/tank

Mix only amount needed to complete job.   Avoid the need to store unused mixtures in

spray tank.