# Split Spoon Test in Shale

# Neq blows/ft and Estimated Equivalent qu

Follow the ASTM procedure for split-spoon sampling.  It will be necessary to have an N60 value of 50 in 6” or less before you can quit.  Current MoDot practice does not use a seating blow in soil and uses 3 seating blows which are not counted for shale or soft rock.

Ex.

Drill No.   G-8690   		Auto Hammer Efficiency   76

| Depth Ft. | Spt Blows/6” | N60 | Neq blows/ft. | Est Equivalent qu, ksf |
| --- | --- | --- | --- | --- |
| 5 | 3-4-40 in 3” | 56 in 9” | 75 | 14.9 |
| 10 | 20 – 40 in 2” | 76 in 8” | 114 | 22.8 |
| 15 | 40 in 2” | 102 in 4” | 306 | 61.2 |

Drill No.   G-8641   		Auto Hammer Efficiency   73

| Depth Ft. | Spt Blows/6” | N60 | Neq blows/ft. | Est Equivalent qu, ksf |
| --- | --- | --- | --- | --- |
| 5 | 3-4-41 in 3” | 55 in 9” | 73 | 14.7 |
| 10 | 20 – 41 in 2” | 74 in 8” | 111 | 22.2 |
| 15 | 41 in 2” | 100 in 4” | 300 | 60.0 |
| 20 | 20-20-21 | 50 | 50 | 10.0 |
| 25 | 20-30-22 | 63 | 63 | 12.6 |

Neq blows/ft = 12*N60 blows/inches penetration

Est. Quivalent qu in ksf = Neq blows/ft. * 0.2