| Project Surveyor’s Name (Print) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | License Number |  |  |  |  |  |  |  |  |  |  |  |  |  | Date of Survey |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enter Project Surveyor Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MO |  | Enter PLS Number |  |  |  |  |  |  |  |  |  |  |  | Enter Date of Survey |  |  |  |  |  |  |  |  |
| Reviewer’s Name (Print) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reviewer’s Name (Signature) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date Reviewed |  |  |  |  |  |  |
| Enter Reviewer Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Enter Date Reviewed |  |  |  |  |  |  |
| Description of Survey |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Enter description of survey |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General Land Surveying Requirements (20 CSR 2030-16.030) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  | NA |  |
| 1. |  | Name of person or entity for whom the survey was made |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 2. |  | Date of the survey |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 3. |  | Lettering 0.08" or larger in height |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 4. |  | Direction of all lines surveyed with directional reference |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 5. |  | North arrow, written scale and graphic scale on each sheet containing graphics. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 6. |  | Horizontal (ground) distances of all lines surveyed in US Feet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 7. |  | All curved lines defined with minimum of two (2) elements. Additional directional component for non-tangential curves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 8. |  | Distance and area dimensions representative of actual precision |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 9. |  | Show or reference source of boundary description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 10. |  | Show property description of parcel created |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  | NA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. |  | Sufficient data to locate the property (distance and direction) within the USPLSS, or within record subdivision |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 12. |  | All controlling corners found and exterior corners found or set identified on plat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 13. |  | Any material variation between measured and record dimensions and/or between measured and lines of possession |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 14. |  | Reference source document(s) for pertinent data obtained in records research |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 15. |  | Accuracy from type of property being surveyed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 16. |  | Plat shall identify title documents for adjoiners as they appear of record (Including their record source) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 17. |  | Statement that survey is in compliance with Missouri Standards for Property Boundary Surveys |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| Title Block Requirements (20 CSR 2030-2.050) The title block must, at a minimum, contain the following information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18. |  | The name of the licensee either as a sole proprietor, partnership, corporation, limited liability company, or other appropriate entity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 19. |  | The licensee’s address and phone number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 20. |  | Name or identification of project |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 21. |  | Address/location of project (city/county and state) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 22. |  | Date prepared |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 23. |  | Space for the licensee’s signature, date and seal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 24. |  | The printed name, discipline, and license number of the person sealing the document |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 25. |  | The printed name, discipline, and certificate of authority number of the corporation as defined in Section 327.011, RSMo. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |

| Use of Missouri State Plane Coordinate System 1983 (20 CSR 2030-16.050) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  | NA |  |
| 26. |  | Name of first or second order control station used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 27. |  | Coordinates of control station (in meters) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 28. |  | Adjustment date and/or epoch date |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 29. |  | Brief statement of method used to obtain position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 30. |  | Grid factor used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| Location of Easements – “When requested” (20 CSR 2030-16.110) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31. |  | Easements shown on subdivision plat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 32. |  | Easements shown on other recorded documents and provided to Surveyor by Client |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| GLO Corner Preservation and Required Filing of Corner Forms (20 CSR 2030-16.110) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33. |  | Permanent monuments at GLO corners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
| 34. |  | The following GLO corners were restored or reestablished and registered with MDA / Land Survey Program |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  | ☐ |  | ☐ |  |
|  |  | Township |  |  |  |  |  |  |  |  |  | Range |  |  |  |  |  |  |  |  |  | Corner Index Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Enter Township |  |  |  |  |  |  |  |  |  | Enter Range |  |  |  |  |  |  |  |  |  | Enter Corner Index No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Enter Township |  |  |  |  |  |  |  |  |  | Enter Range |  |  |  |  |  |  |  |  |  | Enter Corner Index No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Enter Township |  |  |  |  |  |  |  |  |  | Enter Range |  |  |  |  |  |  |  |  |  | Enter Corner Index No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Enter Township |  |  |  |  |  |  |  |  |  | Enter Range |  |  |  |  |  |  |  |  |  | Enter Corner Index No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Enter Township |  |  |  |  |  |  |  |  |  | Enter Range |  |  |  |  |  |  |  |  |  | Enter Corner Index No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |