# UTILITY FIELD INSPECTION CHECKLIST

INSTRUCTIONS: This checklist is intended to assist district utility staff responsible for field inspection of utility adjustments to allow for highway construction. The list may not be all encompassing.

| Route: |  |
| --- | --- |
| County: |  |
| Job No.: |  |

| ITEMS FOR INSPECTION ON UTILITY ADJUSTMENTS |  |
| --- | --- |
| Read the agreement. Study the estimate, plan, and schedule. Become completely familiar with the work involved in the adjustment. |  |
| Check whether Notice to Proceed (NTP) has been given. |  |
| Check to ensure necessary permits have been obtained. |  |
| Check proposed underground utilities to avoid conflict of elevations with highway installations and check to be sure the utilities are installed at the correct elevation. |  |
| Check vertical clearance of overhead utility installations. For utilities crossing over travelways, the minimum vertical clearance shall be not less than required by the National Electric Safety Code and in no case less than 18 feet as required by Commission Policy. |  |
| Check to ensure minimum horizontal clearances are obtained between utility facilities and highways or railroads. |  |
| Check the placement of poles, towers, and similar above ground facilities to ensure that established policies are observed. |  |
| Check the encasement of underground lines such as water lines and petroleum product lines is in accordance with the plans. |  |
| Ensure manhole grades are accurately correlated with proposed highway construction. |  |
| Require all utility excavation areas that might have a critical influence upon highway construction to be backfilled in accordance with requirements established by the agreement. |  |
| Check that proper documentation of disposal is provided by the utility owner or utility owner’s contractor. |  |
| Record the location of the adjusted facilities on the final plans. |  |
| Maintain all records of the adjustment as noted in EPG 643.X.X Documentation. |  |
| ITEMS FOR INSPECTION OF REIMBURSABLE UTILITY ADJUSTMENTS ONLY |  |
| - Begin recording Daily Utility Reports (C-9s) as soon as the utility owner or utility owner’s contractor begins the adjustment, and do so every day work is being done or weekly if no work is being done. |  |
| Check whether the utility owner is doing work with its own forces or by a contract having MoDOT approval as defined in the agreement. |  |
| Observe and record the condition of retired materials to assure that proper salvage value is identified. The utility owner must inform the district utilities staff of the time and place the material will be available for inspection. Both the utility owner and MoDOT should examine the material together. |  |