# 751.50 Standard Detailing Notes

………………………………………………………………………………………………………………………………………………………………….

(K1.5.1) Use for Bridge Approach Slab (Major Road).

The reinforcing steel in the bridge approach slab and the sleeper slab shall be continuous. The transverse reinforcing steel may be made continuous by providing a minimum lap splice of 29 inches for #5 bars and 44 inches for #6 bars, or by mechanical bar splice.

(K1.5.2) Use for Bridge Approach Slab (Minor Road).

The reinforcing steel in the bridge approach slab shall be continuous. The transverse reinforcing steel may be made continuous by providing a minimum lap splice of 23 inches for #4 bars, or by mechanical bar splice.

………………………………………………………………………………………………………………………………………………………………….